# VERIZON EMS NBI

Northbound Interface Specification

Integration Guide for Verizon OSS Platform

Document Version:	3.1 .5

Date:	March 8, 2026

Status:		Production Release Author:	Verizon Network Integration Team

# Table of Contents

- Introduction

- Fault Management (FM)

- Performance Management (PM)

- Configuration Management (CM)

- FTP/SFTP Configuration

- Security & Authentication

- Error Codes Reference

- Appendix

# Introduction

  - Overview

The Verizon Element Management System (EMS) Northbound Interface (NBI) provides a standardized integration point between network element management and Verizon's Operations Support Systems (OSS). This specification document outlines the comprehensive interface requirements for Fault Management (FM), Performance Management (PM), and Configuration Management (CM) integration within Verizon's infrastructure.

This interface leverages a dual-mode architecture: RESTful JSON APIs for real-time transactional operations (Configuration Management, Active Alarm retrieval) and secure file-based protocols (SFTP/FTPS) for high-volume data transfers (Performance statistics, Historical alarm data).

  - Scope

This specification covers the following areas:

    - Fault Management: Real-time alarm notification, active alarm synchronization, alarm acknowledgment, and escalation procedures.

    - Performance Management: KPI collection scheduling, statistical data retrieval, and performance threshold monitoring.

    - Configuration Management: Device provisioning, topology discovery, inventory management, and configuration parameter updates.

    - File Transfer: Secure file transfer protocols, port specifications, and data retention policies.

  - Architecture Overview

The Verizon NBI architecture adheres to TMF6xx Open API standards and implements JSON over HTTPS for all REST communications. The system architecture guarantees stateless operations, 99.99% availability SLA, and encrypted data transmission using TLS 1 .3 exclusively.

# Fault Management (FM)

The Verizon Fault Management interface enables comprehensive network health monitoring through OSS integration. It operates in dual modes: synchronous REST API polling for active alarm states and asynchronous push notifications (Webhooks/Kafka streams) for real-time event delivery. This section details REST API functionality for alarm lifecycle management.

  - FM REST API Endpoints

| Method | Endpoint | Description | Request/Response Payload |
| --- | --- | --- | --- |
| GET | /api/vzw/v2/faultmgmt/alarms | Fetches all active network alarms. Supports advanced filtering by severity level, network element ID, alarm type, and temporal range. | Req: Query Params (severity, source) Res: JSON Array of Alarm Objects |
| GET | /api/vzw/v2/faultmgmt/alarms/{alarmId} | Returns comprehensive details for a single alarm instance including root cause analysis data. | Req: Path Param {id} Res: Single Alarm Object JSON |
| POST | /api/vzw/v2/faultmgmt/alarms/{alarmId}/acknowledge | Acknowledges an alarm to indicate operator awareness and assignment for resolution. | Req: JSON { "userId": "string", "comment": "string" } Res: 200 OK, Updated Alarm Object |
| POST | /api/vzw/v2/faultmgmt/alarms/{alarmId}/close | Manually closes an alarm when automatic clearance is unavailable or administrative closure is required. | Req: JSON { "userId": "string", "reason": "string" } Res: 200 OK |
| GET | /api/vzw/v2/faultmgmt/history | Queries historical alarm records. For bulk historical data exceeding 1 0,000 records, utilize SFTP file export. | Req: Query Params (startTime, endTime) Res: JSON Array of Historical Alarms |

  - FM Sequence Diagram

|

|

|

|

|

|

|

| <

|

|

|

|

|

|

|

|

|

|

| <

|

1.

GET /api /vzw/v2/faultmgmt/alarms?severi ty=MAJOR

>|

| 2. Query Acti ve Alarms

|	>|

| <	|

3.

200 OK

( JSON Array)

|

|

|

|

4. POST /api /vzw/v2/faultmgmt/alarms/{i d}/acknowledge

{operatorId: " vz_ops_001"}

>|

|

|

|

| <

|

5.

6.

Update Alarm State

>|

State Confi rmed

|

7. 200 OK

( Ack Success) |

|

|

The sequence diagram below demonstrates the interaction flow between Verizon OSS and EMS NBI for alarm acknowledgment processing.

| Veri zon | OSS | Veri zon EMS NBI | Network Element |
| --- | --- | --- | --- |
| ¦ |  | ¦ | ¦ |

  - Technical Details

    - Alarm Severity Levels: CRITICAL (P1 ), MAJOR (P2), MINOR (P3), WARNING (P4), INFO (P5).

    - Event Correlation: Each alarm contains a `vzwCorrelationToken` to associate clearing events with their originating alarm instances.

    - Health Check: OSS systems must implement periodic health monitoring via `GET

/api/vzw/v2/system/health` at 30-second intervals.

# Performance Management (PM)

Verizon Performance Management facilitates the aggregation of network metrics (counters, gauges, KPIs) from distributed network elements. Given the massive data volumes, the PM interface utilizes file-based bulk transfer as the primary mechanism. REST APIs govern measurement job lifecycle and on-demand report generation.

  - PM REST API Endpoints

| Method | Endpoint | Description | Request/Response Payload |
| --- | --- | --- | --- |
| POST | /api/vzw/v2/performance/jobs | Initiates a new performance collection job with configurable granularity (5min, 1 5min, 1 hour, 24hour) and resource filtering. | Req: JSON Job Definition Res: 201 Created, Job ID |
| GET | /api/vzw/v2/performance/jobs | Retrieves a catalog of all currently scheduled PM collection jobs. | Req: None Res: JSON Array of Jobs |
| DELETE | /api/vzw/v2/performance/jobs/{jobId} | Terminates and removes a specific performance measurement job. | Req: None Res: 204 No Content |
| GET | /api/vzw/v2/performance/datafiles | Enumerates available PM data files prepared for SFTP retrieval. | Req: Query Params (startDate, endDate, elementId) Res: JSON Array (filePaths, fileSizes, checksums) |

  - PM Sequence Diagram

| 1. POST /api /vzw/v2/performance/j obs

|	{granulari ty: " 15mi n", scope: " regi on_east"}

| 	>|

|	|

| 2. 201 Created |

|	{j obId:  " VZ-PM-20260308-001"}

| <	|

|

|

|

|

|

|

|

|

|

. . .

|

( 15 mi nute collecti on i nterval) . . .

|

|

|

|

| <

|

3.

4.

Generate CSV Fi les

>|

Fi le Ready Noti fi cati on

|

5.  GET  /api /vzw/v2/performance/datafi les?j obId=VZ-PM-20260308-001

| 	>|

|

|

|

6. 200 OK

|

|

{path: "/pm_data/2026/03/08/"}

| <	|

|

|

|

|

|

| <

|

7.

|

SFTP GET /pm_data/2026/03/08/*. csv. gz

>|

|

8.

Fi le Transfer ( Bi nary Stream)

|

|

The following sequence illustrates PM job creation and subsequent data file retrieval workflow.

| Veri zon | OSS | Veri zon EMS NBI | SFTP Fi le Server |
| --- | --- | --- | --- |
| ¦ |  | ¦ | ¦ |

  - Technical Details

Note: Verizon PM data files are retained on EMS servers for 45 days. OSS must retrieve and archive files within this retention period to prevent data loss.

- File Formats: CSV (Comma Separated Values - Default) or 3GPP XML (TS 32.435

- Optional).

- Naming	Convention:

`VZW_PM_{REGION}_{NE_ID}_{YYYYMMDD_HHMM}_{GRAN}.csv.gz`

- Compression: All performance files utilize GZIP compression (level 6) to optimize network bandwidth.

# Configuration Management (CM)

Verizon Configuration Management delivers comprehensive capabilities for service activation, network element parameter modification, and inventory synchronization. This interface operates in fully synchronous mode with ACID transaction guarantees.

  - CM REST API Endpoints

| Method | Endpoint | Description | Request/Response Payload |
| --- | --- | --- | --- |
| GET | /api/vzw/v2/config/inventory | Retrieves complete network inventory topology including Sites, Racks, Shelves, Cards, and Port-level details. | Req: Query Params (depth, rootId) Res: JSON Inventory Tree |
| PATCH | /api/vzw/v2/config/elements/{elementId} | Updates configuration parameters of a specific Managed Network Element (MNE). | Req: JSON Patch Document (RFC 6902) Res: 200 OK, Updated Object |
| POST | /api/vzw/v2/config/backup/execute | Initiates an immediate configuration snapshot backup for specified network elements. | Req: JSON { "neId": "string" } Res: 202 Accepted (Async Token) |
| POST | /api/vzw/v2/config/services/provision | Activates a new service instance or logical network interface. | Req: JSON Service Model Res: 201 Created |

  - CM Sequence Diagram

The sequence diagram below illustrates a typical	configuration modification workflow.

| Veri zon | OSS | Veri zon EMS NBI | Network Element |
| --- | --- | --- | --- |
| ¦ |  | ¦ | ¦ |

| ¦ ¦ | ¦ ¦ | 3. | NETCONF | >¦ |
| --- | --- | --- | --- | --- |
| ¦ | ¦ |  |  | ¦ |
| ¦ ¦ | ¦ ¦ < | 4. | OK | ¦ |
| ¦ | ¦ |  |  |  |

| 1. PATCH /api /vzw/v2/confi g/elements/NE-VZW-001

|	{" transmi tPower": 43, " antennaGai n": 18}

| 	>|

|	|

|	| 2. Vali date Parameter Constrai nts

|	| ----( i nternal)	|

|	|

|	| 5. Update Confi gurati on DB

|	| ----( i nternal)	|

|	|

| 6. 200 OK	|

|	{" status": " CONFIG_APPLIED", " txId": " VZ-CFG-78291"}

| <	|

|	|

FTP/SFTP Configuration

For high-volume data transfers (Performance statistics, Configuration backups, System logs), the Verizon EMS NBI implements secure file transfer protocols. SFTP (SSH File Transfer Protocol) is the mandatory standard for all production deployments. FTP protocol is deprecated and disabled in Verizon environments.

  - Port and Protocol Specifications

| Protocol | Port | Direction | Description | Security Recommendation |
| --- | --- | --- | --- | --- |
| SFTP (SSH) | 2222 (Standard) | Verizon OSS -> EMS (Pull) | Primary secure channel for all PM/CM/Log file retrieval operations. | Mandatory (AES-256 Encrypted) |
| FTP | 21 | N/A | DEPRECATED - Not supported in Verizon production networks. | Prohibited (Security Risk) |
| FTPS | 990 | Verizon OSS -> EMS (Pull) | FTPS with explicit TLS. Backup protocol for SFTP compatibility issues. | High (TLS 1 .3 Encrypted) |

  - Directory Structure

Upon SFTP authentication, users are restricted to a chrooted directory structure as follows:

/veri zon_ems/

├── performance_data/

│

│

│

│

│

│

├── 5mi n/

│

└── {YYYY}/{MM}/{DD}/

├── 15mi n/

│

└── {YYYY}/{MM}/{DD}/

└── hourly/

└── {YYYY}/{MM}/{DD}/

├── confi g_backups/

│	└── {regi on}/

│	└── {ne_i d}/

├── alarm_hi story/

│	└── {YYYY}/{MM}/

└── system_logs/

└── {log_type}/

  - Configuration Instructions

To establish SFTP connectivity, Verizon OSS administrators must provision ED2551 9 or RSA 4096-bit SSH public keys. Password-based authentication is strictly prohibited for all automated system accounts.

sftp

-P

2222

-i

/verizon/keys/vzw_ems_rsa

-o

"StrictHostKeyChecking=yes" vzw_oss_svc@ems-nbi.verizon.internal

Example SFTP Connection:

# Security & Authentication

  - Authentication Mechanism

The Verizon EMS NBI REST API implements OAuth 2.0 (Client Credentials Flow) with mutual TLS (mTLS) for authentication.

- The Verizon OSS client requests an access token from the Identity Provider:

POST /api/vzw/v2/auth/token

.

- The response contains a signed JWT (JSON Web Token) with 30-minute validity.

All	API	calls	require	the	header:	Authorization: Bearer {token} X-Verizon-Client-ID: {clientId} .

and

  - Transport Security

    - TLS Version: TLS 1 .3 is mandatory for all connections. TLS 1 .2 and below are disabled and blocked.

    - Cipher Suites: Only FIPS 1 40-2 compliant cipher suites are permitted (e.g., TLS_AES_256_GCM_SHA384, TLS_CHACHA20_POLY1 305_SHA256).

  - Rate Limiting

To ensure system stability, the Verizon NBI enforces a rate limit of 1 00 requests per second per OSS client instance. Rate limit violations return HTTP 429 with Retry-After header.

# Error Codes Reference

The Verizon NBI API returns standard HTTP status codes with Verizon-specific error codes in the response body for detailed troubleshooting.

| HTTP Code | Internal Code | Description | Resolution |
| --- | --- | --- | --- |
| 400 | VZW_ERR_4001 | Invalid Request Parameter | Verify payload schema against API documentation. |
| 401 | VZW_ERR_401 0 | Authentication Token Expired | Request new token from /api/vzw/v2/auth/token endpoint. |
| 403 | VZW_ERR_4030 | Insufficient Permissions | Client lacks required RBAC role for this operation. |
| 404 | VZW_ERR_4040 | Resource Not Found | Requested network element or resource ID does not exist. |
| 500 | VZW_ERR_5000 | Internal System Error | Contact Verizon EMS Support (Tier 3) with transaction UUID. |

# Appendix

- Acronyms

EMS: Element Management System NBI: Northbound Interface

OSS: Operations Support System NE: Network Element

FM: Fault Management

PM: Performance Management CM: Configuration Management

- Date and Time Format

2026-03-08T19:45:00Z

All timestamps in Verizon API responses and file naming follow ISO 8601 format in UTC timezone.

Example:

©	2026	Verizon	Communications	Inc.	All	rights	reserved.	Confidential	and Proprietary Information.